#include <array>

// ==============================================================================

// GOVERNOR AGGREGATOR (M-of-N Voting Logic)

// Optimized for Multi-Diagnostic Sensor Fusion

// ==============================================================================

template<size_t NumChannels, size_t VoteThreshold>

class GovernorAggregator {

static_assert(VoteThreshold <= NumChannels, "Threshold cannot exceed channel count.");

std::array<OmegaGovernor, NumChannels> governors;

struct GlobalStatus {

SystemMode aggregate_mode = SystemMode::INIT;

bool global_trip_actuated = false;

bool any_supervisory_warning = false;

uint16_t trip_bitmask = 0; // Identifies which specific sensors are firing

uint16_t fault_bitmask = 0;

};

public:

GlobalStatus tick(const std::array<double, NumChannels>& signals, bool watchdog_ok) {

GlobalStatus status;

int trip_votes = 0;

int fault_count = 0;

for (size_t i = 0; i < NumChannels; ++i) {

auto payload = governors[i].tick(signals[i], watchdog_ok);

// 1. Track individual Trip states

if (payload.fast_trip_actuation) {

status.trip_bitmask |= (1 << i);

trip_votes++;

}

// 2. Track individual Hardware Faults

if (payload.mode == SystemMode::HARDWARE_FAULT) {

status.fault_bitmask |= (1 << i);

fault_count++;

}

// 3. Track Supervisory Warnings (Fracture/Drift)

if (payload.supervisory_warning) {

status.any_supervisory_warning = true;

}

}

// --- VOTING LOGIC ---

// We only actuate a global dump if M-of-N sensors agree.

if (trip_votes >= VoteThreshold) {

status.global_trip_actuated = true;

status.aggregate_mode = SystemMode::TRIPPED;

}

else if (fault_count == NumChannels) {

status.aggregate_mode = SystemMode::HARDWARE_FAULT;

}

else if (status.any_supervisory_warning) {

status.aggregate_mode = SystemMode::ALARM_FRACTURE;

}

else {

status.aggregate_mode = SystemMode::MONITORING;

}

return status;

}

void reset_all() {

for (auto& g : governors) g.reset();

}

};